# Research Paper Summaries

16 papers, one line each. Full detail is in research-papers-summary.docx.

| Paper | Authors | Year | Publisher | Relevance |
| --- | --- | --- | --- | --- |
| A Complete Multiagent Framework for Robust and Adaptable Dynamic Job Shop Scheduling | Ning Liu, Mohamed A. Abdelrahman, and Srini Ramaswamy | 2007 | IEEE Xplore | This paper offers a concrete decentralized negotiation mechanism (repeated combinatorial auctions between job and machine agents, with no central coordinator) that directly parallels the project's Contract-Net-based negotiation between agents for production capacity. |
| A Multi-Agent System Framework for Manufacturing Planning and Control | T. N. Wong, C. W. Leung, and H. P. Tang | 2008 | IEEE Xplore | This paper is one of the closest direct matches to the project: it uses a FIPA-compliant contract-net-style negotiation protocol between autonomous part and machine agents to decide production plans and schedules, with a supervisor agent providing light-touch coordination. |
| Agent-Based Distributed Manufacturing Process Planning and Scheduling: A State-of-the-Art Survey | Weiming Shen, Lihui Wang, and Qi Hao | 2006 | IEEE Xplore | As a broad, well-cited survey, this paper is the strongest single background reference for the project's negotiating-agents design: it catalogs how prior systems structure agents (by part, machine, or resource), which negotiation protocols they use (contract net and market/auction-based bidding in particular), and what architectural trade-offs (hierarchical vs. |
| Agent-Based Interaction Protocols and Topologies for Manufacturing Task Allocation | Mohammad Owliya, Mozafar Saadat, Guiovanni G. Jules, Mahbod Goharian, and Rachid Anane | 2013 | IEEE Xplore | This paper is a direct reference for the negotiation layer of the project: it explicitly compares Contract-Net-based negotiation topologies (star vs. |
| Decentralized Tabu Searches in Multi Agent System for Distributed and Flexible Job Shop Scheduling Problem | Bilel Marzouki, Olfa Belkahla Driss, and Khaled Ghédira | 2017 | IEEE Xplore | This paper is a solid supporting reference: it shows a working example of independent agents (one per factory, one per job) coordinating through message-passing to solve a distributed scheduling problem without central control, which mirrors the negotiating-agent structure planned for this MRP II project, though it uses tabu search optimization rather than a Contract Net negotiation protocol. |
| Distributed Manufacturing Scheduling Using Intelligent Agents | Weiming Shen | 2002 | IEEE Xplore | This paper is a foundational primer for the decentralized MRP II project: it explains, in accessible terms, why representing machines/resources as negotiating agents beats a single central planner for dynamic manufacturing, and it introduces the Contract Net Protocol that the project's simplified negotiation mechanism is directly based on. |
| Dynamic Resource Task Negotiation to Enable Product Agent Exploration in Multi-Agent Manufacturing Systems | Ilya Kovalenko, Daria Ryashentseva, Birgit Vogel-Heuser, Dawn Tilbury, and Kira Barton | 2019 | IEEE Xplore | This paper gives a working, tested negotiation pattern between product/job agents and resource/machine agents that is directly analogous to the project's negotiating agents for materials and production capacity, and it shows how a Contract-Net-style bidding process can operate over a local, neighbor-only network rather than requiring a full global factory model — exactly the "no single global view" decentralization the MRP II project aims for. |
| Multi-Agent Reinforcement Learning for Real-Time Dynamic Production Scheduling in a Robot Assembly Cell | Dazzle Johnson, Gang Chen, and Yuqian Lu | 2022 | IEEE Xplore | This paper is a useful advanced reference showing decentralized agents making independent scheduling decisions from local information alone, which lines up with the decentralized philosophy of the negotiating-agents project. |
| Supply Chain Management: A Multi-Agent System for Collaborative Production Planning | Wilhelm Dangelmaier, Jens Heidenreich, and Ulrich Pape | 2005 | IEEE Xplore | This paper directly supports the decentralized MRP II project's core idea: independent agents (material, process, resource) that negotiate and re-synchronize production plans across organizational boundaries without central control. |
| The Methods of Improving the Manufacturing Resource Planning (MRP II) in ERP | Wenchao Jiang and Jingti Han | 2009 | IEEE Xplore | This paper is a good grounding reference for what MRP II actually plans (production scheduling, material requirements, lead-times, safety stock) and why its traditional fixed-parameter approach is brittle. |
| A market-based multi-agent system model for decentralized multi-project scheduling | Giuseppe Confessore, Stefano Giordani, and Silvia Rismondo | 2007 | SpringerLink | This is a strong negotiation-mechanism reference for the project: it demonstrates a concrete, working alternative to Contract Net for resolving conflicts over shared resources, using iterative combinatorial auctions where competing agents bid for bundles of capacity and a coordinator agent clears the market, directly relevant to how supplier/production-capacity negotiation could be structured. |
| Agent-Based Systems for Intelligent Manufacturing: A State-of-the-Art Survey | Weiming Shen and Douglas H. Norrie | 1999 | SpringerLink | As a foundational survey, this paper maps out the broader landscape the MRP II negotiating-agents project sits within, and its discussion of Contract Net and market-driven negotiation protocols for scheduling and resource allocation directly supports the project's plan to use a simplified FIPA Contract Net protocol for material, delivery, and capacity negotiation. |
| An agent-based negotiation approach to integrate process planning and scheduling | T. N. Wong, C. W. Leung, K. L. Mak, and R. Y. K. Fung | 2006 | Taylor & Francis | This paper is a strong direct reference for the negotiation layer of the project: it shows a concrete, modified Contract Net Protocol where part and machine agents bid for manufacturing operations, plus a currency/pricing function to score bids. |
| Auction-theoretic coordination of production planning in the supply chain | Kadir Ertogral and S. David Wu | 2000 | Taylor & Francis | This paper offers a concrete auction/pricing mechanism for coordinating production planning decisions across autonomous facilities without central visibility, directly relevant to how the project's agents might negotiate production targets between suppliers and customers. |
| Integrated shop floor control using autonomous agents | Grace Yuh-Jiun Lin and James J. Solberg | 1992 | Taylor & Francis | This is one of the foundational papers for the project's core idea: independent agents for parts and manufacturing resources negotiating through Contract-Net-style bidding and a market pricing mechanism, adapting automatically to disruptions like machine failures. |
| Modelling and simulation of MRP II activities in multi agent systems | S. Turgay, C. Kubat, and H. Taşkın | 2007 | Taylor & Francis | This is the closest structural match to the target project: it decomposes MRP II (order handling, purchasing, and manufacturing activities) into distinct communicating agents and evaluates their timing and coordination through simulation. |