# Research Paper Summaries

# IEEE Xplore

## A Complete Multiagent Framework for Robust and Adaptable Dynamic Job Shop Scheduling

- Authors: Ning Liu, Mohamed A. Abdelrahman, and Srini Ramaswamy

- Year / Venue: 2007, IEEE Transactions on Systems, Man, and Cybernetics, Part C, 37(5), 904-916

- DOI: 10.1109/TSMCC.2007.900658

In plain terms

Job shops (factories where different jobs need different sequences of machines) constantly face surprises: new jobs arrive, machines break, jobs take longer or shorter than planned. This paper builds a fully decentralized scheduling system where every job and every machine is its own software agent, and they figure out the schedule together through repeated small auctions rather than one central planner recomputing everything. The goal is a system that stays stable and performs well even when the future is uncertain and disruptions keep happening.

Approach

- Each machine agent acts as an "auctioneer" and each job agent acts as a "bidder"; machines post prices for time slots and jobs submit bids for the time slots they want, repeated over several rounds until prices settle (based on an optimization technique called Lagrangian relaxation).

- Combines this auction process with a "rolling time horizon" — instead of planning the whole future at once, the system only plans a limited window ahead and slides that window forward as time passes, keeping computation manageable.

- Purely distributed architecture: there is no master agent and no shared global schedule; each machine only needs to know about the jobs currently bidding on it.

- Tests four scenarios with different levels of foreknowledge about when jobs will arrive: no information, perfect knowledge of arrivals, partial knowledge, and a "good forecast" using probability estimates.

- Benchmarks the framework against seven standard dispatching rules (e.g., earliest due date, shortest processing time) on five classic job-shop scheduling test problems.

Key findings

- The multiagent auction framework (called CMAF) produced more stable, consistent scheduling performance than traditional dispatching rules, even though some individual rules occasionally scored better on raw job-lateness numbers.

- The system handled unpredictable disruptions (job arrivals, breakdowns, timing changes) gracefully because agents only need local information, not a full global picture.

- Having better forecast information about future job arrivals ("good forecast") improved robustness more than either perfect or partial forecasts did, showing that even imperfect predictive information is valuable.

- The size of the planning window (how far ahead the rolling horizon looks) affects performance, and the ideal window size varies by scenario, meaning it needs to be tuned per situation.

Relevance to this project

This paper offers a concrete decentralized negotiation mechanism (repeated combinatorial auctions between job and machine agents, with no central coordinator) that directly parallels the project's Contract-Net-based negotiation between agents for production capacity. Its emphasis on robustness under disruption and its rolling-horizon technique for balancing planning quality against computation cost are directly applicable to how the project's agents should reschedule and rebid as jobs, materials, and supplier deliveries change unexpectedly.

## A Multi-Agent System Framework for Manufacturing Planning and Control

- Authors: T. N. Wong, C. W. Leung, and H. P. Tang

- Year / Venue: 2008, 7th World Congress on Intelligent Control and Automation

- DOI: 10.1109/WCICA.2008.4592959

In plain terms

Factories are traditionally run by one central controller deciding everything, which is simple but inflexible and slow to react to problems like a broken machine. This paper builds a system where independent software agents representing parts and machines negotiate with each other to figure out how and when things get made, while a supervisor agent only steps in if the local decisions conflict with the factory's overall goals. It also gives each agent both a "thinking" layer for planning and a "reflex" layer for immediate real-time reactions, so the system stays fast even under disruption.

Approach

- Proposes a hybrid multi-agent framework with part agents (PAs), machine agents (MAs), and one supervisory agent (SA); PAs and MAs negotiate directly (heterarchical), while the SA monitors and intervenes only when local decisions clash with system-wide goals (hierarchical).

- Each agent has a two-layer internal structure: a "planning layer" for reasoning/negotiation decisions and a "reactive layer" for fast, rule-based real-time control of the physical hardware (robots, CNC machines).

- Designs a Hybrid Contract Net Protocol (HCNP), an extended version of the standard FIPA-style contract net negotiation, that supports many-to-many bidding between parts and machines plus supervisor oversight.

- Bids submitted by machine agents during negotiation are scored using a "currency function" that weighs criteria such as processing time and due dates.

- Implements the framework on the JADE agent platform (which follows FIPA standards) and tests it on a small example with 2 part types and 4-5 machines, including alternative routings represented as an AND/OR graph.

- Demonstrates dynamic rescheduling: when a machine breaks down mid-schedule, affected operations are identified and renegotiated automatically rather than requiring a full manual replan.

Key findings

- The hybrid negotiation approach successfully produces a full process plan and schedule for the test case, generating a Gantt-style negotiation result covering all machines and operations.

- When a simulated machine breakdown occurs, the system correctly identifies both directly and indirectly affected operations and reschedules only those, without needing to redo the whole plan.

- Separating agents into a planning layer and a reactive layer keeps hardware control decoupled from decision-making logic, so hardware can be swapped without redesigning the whole agent.

- The supervisory agent's occasional intervention lets the system balance local (self-interested) agent decisions against global objectives such as minimizing makespan or balancing machine load.

Relevance to this project

This paper is one of the closest direct matches to the project: it uses a FIPA-compliant contract-net-style negotiation protocol between autonomous part and machine agents to decide production plans and schedules, with a supervisor agent providing light-touch coordination. Its hybrid contract net protocol, currency-function bid evaluation, and dynamic rescheduling on disruption are strong reference points for designing the negotiation logic and agent architecture for decentralized MRP II.

## Agent-Based Distributed Manufacturing Process Planning and Scheduling: A State-of-the-Art Survey

- Authors: Weiming Shen, Lihui Wang, and Qi Hao

- Year / Venue: 2006, IEEE Transactions on Systems, Man, and Cybernetics, Part C, 36(4), 563-577

- DOI: 10.1109/TSMCC.2006.874022

In plain terms

Manufacturing process planning (deciding how to make a product) and scheduling (deciding when and on which machine each step happens) are normally treated as separate, centralized problems, which produces plans that quickly become outdated once the real, changing shop floor is factored in. This survey reviews years of research on using software agents instead, where independent agents representing machines, parts, and resources negotiate to plan and schedule work in a distributed way, and it also covers work that tries to combine planning and scheduling into one integrated agent-based process.

Approach

- Explains why manufacturing scheduling is computationally very hard (NP-hard, combinatorial explosion) and becomes even harder when process planning and scheduling are combined or when real-time disruptions are included.

- Surveys traditional (non-agent) approaches to process planning and scheduling, including heuristics, genetic algorithms, simulated annealing, and Petri nets, as context before turning to agent-based methods.

- Reviews numerous agent-based process-planning systems and agent-based scheduling systems, classifying scheduling agents by protocol type (e.g., Contract Net Protocol variants, market/auction-based bidding, voting mechanisms).

- Surveys systems that integrate process planning and scheduling together (rather than as two separate stages), covering both centralized optimization methods and agent-based distributed methods.

- Discusses major design issues across all these systems: how agents are decomposed (by function vs. by physical entity), how agents represent knowledge/negotiate, and system architectures (hierarchical, federated, or fully autonomous).

- Reviews negotiation/coordination mechanisms in depth, especially the Contract Net Protocol (its bidding process and known improvements) and market-based/auction approaches, plus hybrids with GAs and simulated annealing.

Key findings

- Agent-based approaches offer real advantages for distributed manufacturing: modularity, reconfigurability, scalability, upgradability, and better robustness/fault recovery compared to centralized planning and scheduling.

- The Contract Net Protocol (an agent broadcasts work, other agents bid, the best bid wins the contract) is the most widely used coordination mechanism in agent-based manufacturing scheduling, though it faces communication-overhead problems at scale, which various later refinements try to fix.

- Federated agent architectures (using facilitators, brokers, matchmakers, or mediators) are increasingly favored over purely hierarchical or fully autonomous designs because they balance coordination overhead against system flexibility.

- Despite much research, no unified theoretical framework or accepted benchmark yet exists for comparing agent-based process-planning-and-scheduling systems, and integrating planning, scheduling, and real-time execution control together remains an open challenge.

Relevance to this project

As a broad, well-cited survey, this paper is the strongest single background reference for the project's negotiating-agents design: it catalogs how prior systems structure agents (by part, machine, or resource), which negotiation protocols they use (contract net and market/auction-based bidding in particular), and what architectural trade-offs (hierarchical vs. federated vs. autonomous) apply. It is a good source for justifying protocol and architecture choices, such as picking a simplified Contract Net Protocol, even though it doesn't propose a single new system itself.

## Agent-Based Interaction Protocols and Topologies for Manufacturing Task Allocation

- Authors: Mohammad Owliya, Mozafar Saadat, Guiovanni G. Jules, Mahbod Goharian, and Rachid Anane

- Year / Venue: 2013, IEEE Transactions on Systems, Man, and Cybernetics: Systems, 43(1), 38-52

- DOI: 10.1109/TSMCA.2012.2192263

In plain terms

When machines and jobs are represented as software agents that need to allocate work amongst themselves, two design choices matter a lot: how the agents are "wired together" (network shape) and what conversation rules they follow to agree on who does what. This paper builds and tests four different combinations of network shape and negotiation rule for factory task allocation, to see which setups actually perform best on measures like completion time, cost, and machine usage.

Approach

- Compares four agent models: a "star" model (one central manager negotiating with all machines using the classic Contract Net Protocol), a peer-to-peer (P2P) model (every machine can also act as a manager, still using Contract Net), a "ring" model (machines pass a task list around in a circle, each grabbing what it can do, no manager), and a "modified ring" (ring plus the ability for agents to message each other and swap in a better offer as the list circulates).

- Builds a Java-based multiagent simulation (using the Repast simulation platform) so all four models can run on the same task and resource data and be compared fairly.

- Defines measurable performance indicators: total time to finish all jobs, total cost (operating cost, lateness penalties, idle-machine cost), and how busy/idle each machine is.

- Tests the models on a real industrial case study (a gas turbine production shop with turning, milling, and assembly stations) plus extra scenarios: no lateness penalty, relaxed deadlines, jobs with no dependencies, and sudden "rush" jobs inserted mid-schedule.

- Also evaluates the models more abstractly by counting messages exchanged, how much decision-making is spread out (decentralization), and how vulnerable each model is to a single agent failing.

Key findings

- The P2P (Contract-Net-based) model and the new modified-ring model consistently performed best overall, beating the plain star and plain ring models on most measures.

- The star model, despite being centralized, still performed reasonably well but is fragile: if the central manager fails, the whole system breaks down.

- The plain ring model was the weakest performer generally, though it handled independent, unrelated tasks well and was less sensitive to sudden rush jobs than P2P.

- No single model won on every measure — the best choice depends on the situation (e.g., how tight deadlines are, whether tasks depend on each other, whether disruptions are frequent), and the modified ring proved to be a strong new alternative to the well-established Contract-Net methods.

Relevance to this project

This paper is a direct reference for the negotiation layer of the project: it explicitly compares Contract-Net-based negotiation topologies (star vs. peer-to-peer) against alternative ring-style protocols for allocating manufacturing tasks, giving concrete evidence about the tradeoffs (robustness, decentralization, communication overhead) of different agent network structures. Its performance framework (time, cost, utilization, communication efficiency, failure resilience) is a useful template for evaluating the project's own Contract Net implementation.

## Decentralized Tabu Searches in Multi Agent System for Distributed and Flexible Job Shop Scheduling Problem

- Authors: Bilel Marzouki, Olfa Belkahla Driss, and Khaled Ghédira

- Year / Venue: 2017, IEEE/ACS International Conference on Computer Systems and Applications, 1019-1026

- DOI: 10.1109/AICCSA.2017.133

In plain terms

This paper tackles the problem of scheduling jobs across several separate factories at once, where each job's operations can run on different machines with different processing times. Instead of solving this with one central planner (as most prior work did), the authors build a system of cooperating software agents that work it out among themselves, aiming to finish all jobs as quickly as possible (minimize the "makespan," i.e., the completion time of the last job).

Approach

- Models the problem as the Distributed and Flexible Job Shop Scheduling Problem (DFJSP): multiple factories, each with multiple machines, and jobs that must be assigned both to a factory and to specific machines within it.

- Uses three types of agents: one Supervisor Agent (builds the initial job assignment and oversees the whole run), several Factory Agents (one per factory, manage scheduling within that factory), and multiple Scheduler Agents (one per job, run the optimization).

- Optimization is done with the Tabu Search metaheuristic, a local-search method that explores nearby schedule variations while keeping a "tabu list" of recently tried moves to avoid cycling back to them.

- Includes a diversification step: if no improvement is found for a while, the Supervisor Agent swaps jobs between factories to escape getting stuck in a poor solution.

- Built using the JADE multi-agent platform in Java and tested on standard scheduling benchmark datasets (Hurink, Brandimarte, and others).

Key findings

- The decentralized tabu search model reached the best-known (optimal) solution in a large share of test cases, improving as more factories were added (e.g., 50% of instances optimal with 2 factories, up to 85% with 4 factories).

- Performance was competitive with, and sometimes better than, existing centralized genetic-algorithm approaches from earlier literature, though it was occasionally slightly worse on some individual test cases.

- On dedicated DFJSP benchmark instances, the model matched or beat prior published approaches in all tested cases.

Relevance to this project

This paper is a solid supporting reference: it shows a working example of independent agents (one per factory, one per job) coordinating through message-passing to solve a distributed scheduling problem without central control, which mirrors the negotiating-agent structure planned for this MRP II project, though it uses tabu search optimization rather than a Contract Net negotiation protocol.

## Distributed Manufacturing Scheduling Using Intelligent Agents

- Authors: Weiming Shen

- Year / Venue: 2002, IEEE Intelligent Systems, 17(1), 88-94

- DOI: 10.1109/5254.988492

In plain terms

This is a tutorial-style survey explaining why scheduling a factory floor is so hard (too many possible combinations, and constant surprises like machine breakdowns or late materials), and why treating each machine, operator, or job as its own "smart agent" that negotiates with the others is a promising alternative to one central scheduler trying to plan everything. It reviews the main design choices researchers had made up to that point and lays out open research problems.

Approach

- Explains why classical (centralized) scheduling methods (search, heuristics, simulated annealing, genetic algorithms, Tabu search) struggle with real-world, constantly-changing shop floors.

- Describes two ways to turn a factory into agents: by function (e.g., a planning agent, a scheduling agent) or by physical resource (a machine, tool, or operator becomes its own agent) — the latter is more common.

- Surveys negotiation methods, centered on the Contract Net Protocol (one agent advertises work, others bid, the best bid wins), plus market/auction-style variants.

- Categorizes system architectures into hierarchical (tree-like, centralized-ish), federated (agents grouped via facilitators/brokers/mediators), and fully autonomous agent networks, plus hybrids.

- Reviews around a dozen real research systems (e.g., YAMS, ADDYMS, AARIA, CAMPS) built between the 1980s-1990s to illustrate these ideas in practice.

Key findings

- Agent-based, negotiation-driven scheduling handles real-time disruptions (cancellations, breakdowns, delays) far better than centralized approaches, at some cost of guaranteed optimality.

- No single architecture or protocol is best for everything; federated/hybrid designs (mixing brokers, mediators, and autonomous agents) tend to balance flexibility with manageability.

- Combining agent negotiation with traditional optimization techniques (genetic algorithms, simulated annealing, constraint satisfaction) looks promising for getting both responsiveness and better-quality schedules.

- The field still lacked (as of 2002) mature negotiation protocols for complex/combinatorial bidding, shared benchmarks, and formal methods for integrating planning, scheduling, and execution control.

Relevance to this project

This paper is a foundational primer for the decentralized MRP II project: it explains, in accessible terms, why representing machines/resources as negotiating agents beats a single central planner for dynamic manufacturing, and it introduces the Contract Net Protocol that the project's simplified negotiation mechanism is directly based on. Its architecture taxonomy (hierarchical/federated/autonomous) is also a useful reference point when deciding how the project's agents should be organized and connected.

## Dynamic Resource Task Negotiation to Enable Product Agent Exploration in Multi-Agent Manufacturing Systems

- Authors: Ilya Kovalenko, Daria Ryashentseva, Birgit Vogel-Heuser, Dawn Tilbury, and Kira Barton

- Year / Venue: 2019, IEEE Robotics and Automation Letters, 4(3), 2854-2861

- DOI: 10.1109/LRA.2019.2921947

In plain terms

In agent-based factories, a "product agent" (representing a physical part) needs to find out what nearby machines ("resource agents") can do for it before deciding what to request next. Existing methods either give the product agent a complete map of the whole factory (too much irrelevant information) or make it ask every single machine (too much communication traffic). This paper proposes a smarter middle ground: machines are linked into a network based on which physical locations they share, and a product agent only needs to ask its nearest neighbor — that machine then passes the request along its network, letting the machines "team up" to figure out and report back a workable path, without the part or a central system needing to know the whole factory.

Approach

- Each resource agent keeps a local model of its own capabilities (states it can produce, actions it can perform, timing) and knows only its "neighboring" resource agents — those with which it physically shares a handoff point.

- A product agent sends a "bid request" describing what it still needs done and a time limit, starting with the last machine that touched its part; that machine tries to satisfy the request itself, or passes ("propagates") a partial bid to its neighbors if it can't.

- Neighboring resource agents can extend an incomplete bid, effectively forming an ad-hoc "team" of machines that together can satisfy the product agent's request, using a bidding process similar to the Contract Net Protocol.

- The product agent collects all the responses ("bids"), merges them into a temporary local model of its surroundings, and uses that to plan the fastest route/action sequence (e.g., using a shortest-path algorithm).

- The approach lets resources join, leave, or change capability at any time by simply updating their neighbor links, so the network self-adjusts without redesigning the whole system.

- Implemented and tested on a real physical demonstrator (a small yogurt-bottling and conveyor system called myJoghurt) using standard agent software (JADE) and industrial controllers.

Key findings

- The exploration process worked reliably on real hardware: resource agents needed roughly 38 milliseconds on average to negotiate and return a bid, and a full round trip for a product agent to get a usable answer took well under 200 milliseconds.

- In a multi-part test with 20 physical items moving through the system, 18 completed their process successfully; the 2 failures were due to a physical/mechanical fault, not the negotiation approach.

- The system correctly and automatically adapted when new machines were added mid-run, letting product agents discover and use the new equipment without any manual reconfiguration.

- Because agents only share information with direct neighbors, the approach reduces communication overhead, avoids single points of failure (no central map of the whole factory), and lets manufacturers control how much capability information is exposed to which product agents.

Relevance to this project

This paper gives a working, tested negotiation pattern between product/job agents and resource/machine agents that is directly analogous to the project's negotiating agents for materials and production capacity, and it shows how a Contract-Net-style bidding process can operate over a local, neighbor-only network rather than requiring a full global factory model — exactly the "no single global view" decentralization the MRP II project aims for. Its real-hardware timing results (bids returned in well under a quarter of a second) also give a useful practical benchmark for how fast a simplified Contract Net negotiation should perform in the project's own agent system.

## Multi-Agent Reinforcement Learning for Real-Time Dynamic Production Scheduling in a Robot Assembly Cell

- Authors: Dazzle Johnson, Gang Chen, and Yuqian Lu

- Year / Venue: 2022, IEEE Robotics and Automation Letters, 7(3), 7684-7691

- DOI: 10.1109/LRA.2022.3184795

In plain terms

This paper looks at a robot assembly line where new jobs keep arriving unpredictably (not all known in advance), and each robot has to decide in real time which job to pick up next. Instead of using a fixed rule or a single central brain to plan for everyone, the authors give each robot its own decision-making "agent" that learns, through trial and error (reinforcement learning), how to make good choices using only what it can see locally, while still cooperating with the other robots to get all jobs done as fast as possible.

Approach

- Models the shop floor as a conveyor belt loop with robots stationed at fixed points; jobs enter randomly over time (following a Poisson arrival pattern) and each robot can only see and grab jobs in a limited "window" of nearby conveyor sections.

- Frames the problem as Multi-Agent Reinforcement Learning: each robot is an agent that observes its local surroundings, chooses an action (accept a job, wait for a better one, decline, or continue), and receives a reward.

- Uses a "centralized training, decentralized execution" setup: agents are trained together using a shared Double DQN (a deep learning algorithm), but once deployed, each robot decides independently using only its own local observations.

- Designs a custom reward function that penalizes idle time, rewards agents for actively working, and rewards a "wait" decision when skipping a nearby job leads to a better outcome later.

- Limits communication between robots to just sharing basic status information (idle/working/etc.), keeping coordination overhead low.

- Benchmarks the learned policy against six simple rule-based scheduling heuristics (e.g., first-come-first-served, shortest processing time, fastest available robot).

Key findings

- The learned multi-agent approach produced shorter overall completion times (makespan) than all six rule-based heuristics, with the advantage growing as job arrival rates increased and as robots were allowed to see further ahead (larger observation window).

- Under low job-arrival rates, performance was similar to the best simple heuristics, but the gap widened noticeably under busier, higher-arrival-rate conditions (up to about 11% improvement in one tested case).

- Statistical tests confirmed the improvements were significant for larger observation windows and higher workloads, though window size mattered less when the shop floor was not very busy.

Relevance to this project

This paper is a useful advanced reference showing decentralized agents making independent scheduling decisions from local information alone, which lines up with the decentralized philosophy of the negotiating-agents project. However, it replaces explicit agent-to-agent negotiation (like Contract Net) with machine-learned decision policies, so it illustrates an alternative, more complex approach that sits outside the project's planned rule-based negotiation core.

## Supply Chain Management: A Multi-Agent System for Collaborative Production Planning

- Authors: Wilhelm Dangelmaier, Jens Heidenreich, and Ulrich Pape

- Year / Venue: 2005, IEEE International Conference on e-Technology, e-Commerce and e-Service

- DOI: 10.1109/EEE.2005.128

In plain terms

In a supply chain made up of separate companies (supplier, producer, customer), a single company controlling everyone else's plans doesn't work because each business wants to keep its own autonomy. This paper proposes representing each part, process, and production resource in the chain as a software agent, and letting these agents talk to each other and adjust their local plans whenever something changes upstream or downstream, such as a delayed delivery. They also build an editor tool so a non-programmer "supply chain engineer" can configure the agents' behavior without writing code.

Approach

- Models each agent using the BDI (Belief-Desire-Intention) framework: beliefs are the agent's knowledge of its plan/conversation state, desires are its tasks/goals, and intentions are the plans it executes.

- Breaks the supply chain down into three agent types based on the MFERT production model: material agents, process agents, and capacity/resource agents, linked by material and information flow, spanning supplier, producer, and customer.

- Defines a negotiation protocol using standard speech-act style messages (propose, counter-propose, accept, deny, reject) built on KQML/KIF message formats, illustrated with a state-chart diagram.

- Local planning logic follows simple "when event, if condition, then action" rules, covering different ordering strategies (fixed lot size, fixed ordering interval, or both variable).

- Presents a worked scenario where a delayed material delivery from a supplier triggers a chain reaction: the material agent adjusts stock, the process agent recalculates capacity needs, the resource agent responds with available capacity, and the change propagates downstream to the next material agent.

- Builds a prototype tool, MASCOPP, with a graphical editor for the user to draw the agent network, configure each agent's properties and rules, and a runtime environment to execute the resulting multi-agent system.

Key findings

- The scenario demonstrates that a local disruption (a reduced incoming delivery) can be absorbed and re-planned across multiple agents automatically, with the new plan propagating through the chain while keeping the overall required output quantity intact.

- Representing collaboration as agent negotiation avoids needing one central authority to control the whole supply chain, preserving each entity's autonomy.

- The graphical MASCOPP editor shows the approach can be made usable by a domain expert without agent-programming skills, since agent behavior is defined through rules and configuration rather than code.

- The paper positions itself relative to other multi-agent supply chain projects (e.g., Agent.Enterprise/DISPOWEB) as focused specifically on decentralized production-plan synchronization rather than price negotiation between companies.

Relevance to this project

This paper directly supports the decentralized MRP II project's core idea: independent agents (material, process, resource) that negotiate and re-synchronize production plans across organizational boundaries without central control. Its propose/counter-propose/accept negotiation protocol and its disruption-propagation scenario (a delayed supplier delivery rippling through material, process, and resource agents) are a useful template for how material and supplier-delivery negotiation could be modeled with a simplified Contract Net-style protocol.

## The Methods of Improving the Manufacturing Resource Planning (MRP II) in ERP

- Authors: Wenchao Jiang and Jingti Han

- Year / Venue: 2009, International Conference on Computer Engineering and Technology

- DOI: 10.1109/ICCET.2009.18

In plain terms

Classic MRP II (Manufacturing Resource Planning) systems plan production using fixed settings, like a constant lead-time and a constant safety stock, that were entered once when the system was set up. The authors argue this is a problem in fast-changing markets because it wastes stock and hides real capacity limits. Their solution is to replace these fixed numbers with five calculation models that update themselves automatically based on actual demand, workloads, and purchasing conditions.

Approach

- Identifies four weaknesses of traditional MRP II: overly long fixed lead-times, fixed safety stock, ignoring capacity limits, and no real time management.

- Proposes five mathematical/statistical models to plug into the MRP II process: a production-scheduling resource-balancing model, a statistical safety-stock forecasting model, a manufacturing lead-time calculation model, a purchase lead-time forecasting model (using a trained neural network), and a resource-balancing model based on project-scheduling techniques (PERT/CPM).

- The neural network model is trained on historical purchase records (factors like order quantity, transport method, distance, supplier workload) to predict how long a purchase will take.

- The PERT/CPM-based model treats the whole manufacturing plan as a project network, finds the critical path, and balances resources without delaying the overall completion time.

- All five models are shown wired into the standard MRP II process flow (main scheduling to material requirements planning to purchase/production orders).

Key findings

- Integrating the models turns lead-time and safety stock from fixed, manually-set numbers into dynamic values that respond to real demand fluctuation and shop-floor conditions.

- A worked example shows the neural-network model producing a specific purchase lead-time (3.3 days) from real purchase-condition inputs, demonstrating the forecasting approach works on sample data.

- The PERT/CPM integration lets managers see the critical path and adjust only non-critical activities, so resource balancing doesn't push out the overall production deadline.

- The result is described as giving MRP II/ERP a "time management" capability it previously lacked, better suited to competitive, fast-moving markets.

Relevance to this project

This paper is a good grounding reference for what MRP II actually plans (production scheduling, material requirements, lead-times, safety stock) and why its traditional fixed-parameter approach is brittle. It does not use software agents or negotiation, so it is background/definitional material rather than a direct technical basis for the negotiating-agents architecture, but it usefully justifies why dynamic, responsive planning (which this project pursues through agent negotiation) is needed.

# SpringerLink

## A market-based multi-agent system model for decentralized multi-project scheduling

- Authors: Giuseppe Confessore, Stefano Giordani, and Silvia Rismondo

- Year / Venue: 2007, Annals of Operations Research, 150, 115-135

- DOI: 10.1007/s10479-006-0158-9

In plain terms

This paper deals with a company running several projects at once, where each project has its own manager and its own tasks, but all the projects have to share one limited resource. Rather than having one central planner decide who gets the resource when, the authors set up a marketplace where each project manager acts as a software agent that "bids" for time slots on the shared resource, and a coordinator agent runs an auction to fairly and efficiently divide it up.

Approach

- Models each project manager as an independent, self-interested agent that only knows about its own project, plus a separate coordinator agent that manages the shared resource.

- Coordination happens through an iterative combinatorial auction: agents bid for bundles of time slots (not just single slots) since a project usually needs several time periods of the resource together.

- At each round, the coordinator (auctioneer) tentatively awards the resource to the bids that maximize its revenue, then raises the price on the bundles it did not sell, prompting losing bidders to submit revised bids next round; the process repeats until every agent's bid is accepted.

- Because the auction and each agent's own bidding decision are both computationally hard (NP-hard) problems, the paper proposes fast approximate (heuristic) algorithms based on a dynamic-programming approach, rather than solving them exactly.

- Tests the whole system on randomly generated multi-project scheduling scenarios of varying size, comparing different heuristic variants against each other and against a centralized planning method.

Key findings

- The heuristic auction algorithms produced solutions close to the optimal (as measured against upper bounds computed separately) while running fast enough to be used repeatedly across auction rounds.

- The fully decentralized, market-based approach delivered project completion times comparable to, and in some cases better than, a traditional centralized scheduling method, despite each agent having only partial, local information.

- Increasing the number of projects competing for the shared resource increased delays and gaps versus the no-conflict baseline, as expected, but the auction mechanism still converged to a workable allocation.

Relevance to this project

This is a strong negotiation-mechanism reference for the project: it demonstrates a concrete, working alternative to Contract Net for resolving conflicts over shared resources, using iterative combinatorial auctions where competing agents bid for bundles of capacity and a coordinator agent clears the market, directly relevant to how supplier/production-capacity negotiation could be structured.

## Agent-Based Systems for Intelligent Manufacturing: A State-of-the-Art Survey

- Authors: Weiming Shen and Douglas H. Norrie

- Year / Venue: 1999, Knowledge and Information Systems, 1(2), 129-156

- DOI: 10.1007/BF03325096

In plain terms

This is an early, broad review of how software "agent" technology was being used to build smarter, more flexible factory systems. It explains why traditional single-brain (centralized) manufacturing planning and control struggles to keep up with fast-changing customer demands and global competition, and surveys around 50 research projects that instead used networks of independent, cooperating software agents to handle tasks like supply chain management, production planning, scheduling, and shop-floor control.

Approach

- Lays out the requirements next-generation manufacturing systems need to meet: integration with partners and suppliers, ability to handle mixed hardware/software, flexibility to add or remove components on the fly, fast reconfiguration, and resilience to individual failures.

- Defines what counts as an "agent" and an "agent-based system" (an autonomous, situated software entity capable of acting on its own to meet its goals), distinguishing this from older, non-autonomous AI techniques.

- Surveys real research projects in three categories: enterprise/supply-chain integration, manufacturing planning/scheduling/control, and Holonic Manufacturing Systems (where agents represent physical "holons" like machines, parts, and workers that cooperate as equals rather than through strict top-down control).

- Compares two ways agents were built: representing business functions (functional decomposition) versus representing physical things like machines or parts (physical decomposition).

- Reviews system organization styles (hierarchical, federation-based with facilitators/brokers/mediators, and fully autonomous peer agents) and discusses negotiation protocols used for coordinating scheduling decisions, including the Contract Net protocol and market-style bidding/auction mechanisms.

- Also touches on practical concerns: dynamic reconfiguration, learning, integrating planning with scheduling, real-time execution monitoring, and the lack of accepted standards at the time (pointing to early efforts like FIPA and KQML).

Key findings

- Agent-based approaches were shown across many projects to improve responsiveness to market changes, enable better resource allocation, and avoid the single point of failure that centralized systems suffer from.

- No single system architecture was best for everything; hierarchical designs are simpler but less flexible, while federation and autonomous-agent designs scale better but are more complex to build and coordinate.

- Negotiation-based coordination, especially the Contract Net protocol and its many variants (including market-driven versions), was already a common and effective way for agents to allocate scheduling and resource decisions without central control.

- The field lacked mature standards and tools at the time, which the authors flagged as a barrier to wider industrial adoption.

Relevance to this project

As a foundational survey, this paper maps out the broader landscape the MRP II negotiating-agents project sits within, and its discussion of Contract Net and market-driven negotiation protocols for scheduling and resource allocation directly supports the project's plan to use a simplified FIPA Contract Net protocol for material, delivery, and capacity negotiation.

# Taylor & Francis

## An agent-based negotiation approach to integrate process planning and scheduling

- Authors: T. N. Wong, C. W. Leung, K. L. Mak, and R. Y. K. Fung

- Year / Venue: 2006, International Journal of Production Research, 44(7), 1331-1351

- DOI: 10.1080/00207540500409723

In plain terms

In a job-shop factory, deciding which machine should make which part (process planning) and when it should happen (scheduling) are normally handled separately, which can produce plans that look fine on paper but don't fit the real shop floor. This paper has software "part agents" (representing jobs to be made) and "machine agents" (representing manufacturing resources) negotiate directly with each other, bidding for time slots, so that routing and scheduling decisions are made together rather than one after the other.

Approach

- Represents each part's possible manufacturing routes and machine alternatives as an AND/OR graph, capturing multiple valid ways to produce the same part.

- Extends the classic Contract Net Protocol (CNP) so that a part agent can announce several tasks at once to many machine agents simultaneously, rather than one task to one contractor at a time.

- Uses a "currency function" -- a formula based on processing time and due-date urgency (critical ratio) -- to convert each operation into a cost value that agents use to evaluate bids.

- Part agents pick the most favorable bid balancing flow-time and production cost; machine agents pick the most favorable request balancing revenue and machine utilization.

- Includes a re-negotiation mechanism: if a deal cannot be reached, the task is re-announced with a slightly higher price to attract a machine.

- Implements the system in Java using the JADE multi-agent platform (which follows FIPA agent-communication standards) and tests it on 24 benchmark job-shop problems against a genetic-algorithm-style baseline.

Key findings

- The negotiation-based agent system (called MAN) reduced average part flow-time by about 23% compared to the baseline symbiotic evolutionary algorithm (SEA).

- Makespan (total time to finish all jobs) improved only slightly on average (about 10%), and was inconsistent across problems -- agents optimizing their own local goals sometimes worked against the overall factory-wide finishing time.

- Negotiation time budget mattered: giving agents too little time to collect bids (1 second) produced clearly worse schedules than giving them more time (5-12 seconds).

Relevance to this project

This paper is a strong direct reference for the negotiation layer of the project: it shows a concrete, modified Contract Net Protocol where part and machine agents bid for manufacturing operations, plus a currency/pricing function to score bids. It's also a useful cautionary example -- the results show that letting agents purely optimize local objectives can hurt system-wide performance, which is worth keeping in mind when designing the negotiation rules for materials, deliveries, and capacity.

## Auction-theoretic coordination of production planning in the supply chain

- Authors: Kadir Ertogral and S. David Wu

- Year / Venue: 2000, IIE Transactions, 32(10), 931-940

- DOI: 10.1080/07408170008967451

In plain terms

When a product is made across several factories (each possibly owned by a different company), it's usually assumed that everyone will share their full cost and capacity details with a central planner -- which is unrealistic. This paper treats each factory as a self-interested agent and uses an auction-style negotiation, where a "price" signal is passed back and forth between supplier and customer factories, to coordinate production plans without anyone needing full visibility into everyone else's data.

Approach

- Starts from a standard centralized production-planning model (the Multi-Level Capacitated Lot Sizing Problem) that decides how much of each item to make and when, across a multi-tier supply chain with a bill-of-materials structure.

- Reformulates the problem so each facility solves its own local version, linked to neighboring facilities only through shared "flow" variables on their supply relationships.

- First tries a standard Lagrangian decomposition (a classic price-based coordination method) but finds it becomes unstable and oscillates rather than converging to a usable plan.

- Designs an alternative auction-theoretic decomposition where a central auctioneer iteratively updates conflict prices and shared production targets based on how much facilities' plans disagree with each other, and facility agents re-solve their own subproblems each round.

- Introduces a formal "fairness" measure to check whether the burden of compromise is spread evenly across facilities rather than dumped on one.

- Tests the mechanism on 300 standard benchmark planning problems, comparing the auction-derived ("coordinated") solution against the true global optimum, a "maximum fairness" solution, and a center-imposed fallback solution.

Key findings

- The auction-based coordination mechanism converged to solutions that were on average about 10% more costly than the true global optimum, a fairly modest sacrifice in solution quality.

- Its solutions were substantially fairer across facilities than the classic centralized optimum or the center-imposed fallback, improving the fairness measure by roughly 28% over the optimal solution on average.

- Plain Lagrangian (price-only) decomposition was shown to be unreliable for this problem because it kept oscillating between extreme facility-level solutions instead of settling on a plan.

Relevance to this project

This paper offers a concrete auction/pricing mechanism for coordinating production planning decisions across autonomous facilities without central visibility, directly relevant to how the project's agents might negotiate production targets between suppliers and customers. Its finding that fully decentralized coordination trades some efficiency for fairness and feasibility is a useful design consideration for balancing negotiation outcomes against pure cost-optimality in the decentralized MRP II system.

## Integrated shop floor control using autonomous agents

- Authors: Grace Yuh-Jiun Lin and James J. Solberg

- Year / Venue: 1992, IIE Transactions, 24(3), 57-71

- DOI: 10.1080/07408179208964224

In plain terms

Factory floors are messy and unpredictable -- machines break down, tools run out, orders change -- so planning everything from one central controller doesn't cope well. This early paper proposes treating every part and every resource (machines, tools, pallets, transporters) as its own independent software agent with its own goals, and letting them negotiate and "trade" with each other like a small marketplace, using a currency and pricing system, to decide who works on what and when.

Approach

- Gives each part agent a flexible process plan (multiple valid operation sequences) plus its own priority, cost, time, and quality objectives, and a budget of fictitious "currency" to spend.

- Gives each resource agent (machine, tool, transporter, pallet, etc.) the ability to set its own price for its services, adjusting that price based on how busy or in-demand it is.

- Runs negotiation as a multi-step, multi-way bidding process (extending the Contract Net Protocol): parts announce tasks, resources bid, parts evaluate and award, and both sides must mutually accept before a commitment is made.

- Adds a "dynamic resource unification" scheme so that when a part needs several resources at once (e.g., a machine and a tool and a transporter), the scarcest/most critical resource drives the matching decision.

- Builds in robustness features: if a computer or agent fails, other agents can detect it and spawn replacement agents so the system keeps running.

- Validates the framework with an object-oriented discrete-event simulation of a small factory, testing scenarios with machine breakdowns and different traffic loads.

Key findings

- Prices charged by machines adjusted automatically in response to breakdowns and demand, e.g., prices rose while a machine was down for repair and load shifted to other machines.

- The system kept operating smoothly through simulated machine failures and changing traffic loads without a central controller intervening.

- Compared with a simpler first-in-first-out data-flow control baseline, the price/negotiation-based approach achieved better individual and overall system performance.

Relevance to this project

This is one of the foundational papers for the project's core idea: independent agents for parts and manufacturing resources negotiating through Contract-Net-style bidding and a market pricing mechanism, adapting automatically to disruptions like machine failures. Its emphasis on decentralized, price-driven coordination under changing shop-floor conditions maps closely onto how materials, deliveries, and production capacity would need to be negotiated in the target decentralized MRP II system.

## Modelling and simulation of MRP II activities in multi agent systems

- Authors: S. Turgay, C. Kubat, and H. Taşkın

- Year / Venue: 2007, Production Planning & Control, 18(1), 25-34

- DOI: 10.1080/09537280600940663

In plain terms

MRP II (Manufacturing Resource Planning) systems handle orders, inventory, scheduling and production planning for a factory, but they are usually built as one big rigid system. This paper breaks an MRP II system for a make-to-order factory into a set of separate software agents (for work orders, inventory, capacity planning, forecasting, etc.) that pass messages to each other, and then simulates how those agents interact over time to see where delays or bottlenecks show up.

Approach

- Splits MRP II functionality into cooperating agents: an MRP II agent manager, work order management agent, inventory management agent, master production schedule agent, capacity planning agent, shop floor control agent, forecast planning agent, and a device agent, among others.

- Defines the databases and information each agent needs (bill-of-materials, routing data, inventory status, manufacturing reports) and maps out the message exchanges between agents.

- Uses a bidding-style mechanism where a device agent broadcasts a task, other agents prepare bids, and the device agent picks the best (lowest-cost) bid and gets an accept/reject reply.

- Models the whole system as a Petri net (a diagram/math tool for tracking how "tokens" move between states) with each agent represented as a place-transition pair, so the flow of information and materials can be tracked with timing.

- Converts the Petri net into a matrix form and simulates it in MATLAB, assigning estimated processing times to each of the 46 identified inter-agent events.

Key findings

- Simulation revealed that certain events -- especially the information exchange between the MRP II agent and the Work Order Manager agent -- took up a disproportionately large share of the timeline, marking them as likely bottlenecks.

- Two specific transitions (numbered 10 and 25 in the event table) showed heavy information flow, pointing to queues and congestion risk at those points.

- The overall approach let the authors identify bottleneck-prone areas of the system before it would actually be built, which is the main practical payoff of the simulation.

Relevance to this project

This is the closest structural match to the target project: it decomposes MRP II (order handling, purchasing, and manufacturing activities) into distinct communicating agents and evaluates their timing and coordination through simulation. It doesn't use a true negotiation/bidding protocol like Contract Net, but its agent breakdown and its use of simulation to expose bottlenecks are directly useful as a template for structuring a decentralized MRP II agent system.